REC ONNECTING THE RIVER LIFFEY T O IT S CITY

LIFFEY CYCLE ROUTE PROJECT

Join us on our journey along the Liffey

RECOMMENDED OPTION

Aerial View of Dublin City along the Liffey

Liffey Cycle Route / Options Assessment Summary

Overall Objective:

To develop a Liffey Cycle Route which provide a safe, continuous & segregated cycle facility in both directions between Phoenix Park / Heuston Station & the Tom Clarke East Link Bridge.

Liffey Cycle Route / Options Assessment Summary

Contents

1. Introduction	2

5. Recommended Option	14

1.1

Background

3

5.1

Section 1 - Phoenix Park to Father Mathews

16

Bridge (Church/Bridge St)

1.2

Overview

3

Section 2 – Father Mathews Bridge to O’Connell Street

5.2

1.3

Previous Study

4

16

1.4	Project Objectives	5

Section 3 – O’Connell St to Matt Talbot Memorial Bridge

5.3

16

1.5

Study Area

5

2. Design Principles and Assumptions	7

Section 4 – Dublin Docklands

5.4

16

2.1

National Cycle Manual

6. Next Stage

8

18

2.2

Current Traffic System

8

6.1

Public Engagement

19

2.3

Environmental Issues

8

6.2

Cost

19

3. Options Considered	10

7. Recommended Option – Scheme Drawings	20

3.1

Cycling Options

11

7.1

Index Map

21

4. Methodology and Appraisal	12

7.2

Route Maps

22

4.1

Sections

13

4.2

Appraisals

13

1

Liffey Cycle Route / Options Assessment Summary

01

INTRODUCTION

2

SECTION 1

Stock image of Bicycle handlebars

Liffey Cycle Route / Options Assessment Summary

  - Background

The Liffey Cycle Route has been an important objective of Dublin City Council [DCC] and the National Transport Authority [NTA] since 2013. Previous studies have been undertaken to determine how best to advance this primary route on the Cycle Network Plan for the Greater Dublin Area. In 2017, DCC and the NTA agreed that an a review of all options previously considered as well as any new options that hadn’t been identified to date should be undertaken.

  - Overview

This document provides an overview of the work undertaken as part of the Liffey Cycle Route – Options Assessment Report.

The information in this document sets out, in summary form:

Wellington Quay at Grattan Bridge - Proposed

Ĥ The objectives of the scheme;

Ĥ The design principles and assumptions;

Ĥ The constraints;

Ĥ The assessment process; and

Ĥ The Recommended Option.

3

SECTION 1

Photomontage of proposed scheme along Wellington Quay to Grattan Bridge showing the Liffey Cycle route following the quay wall, a new boardwalk outside the quay wall meeting Grattan Bridge, a continuous bus and traffic lanes along the Quays, and enhanced footpaths.

Liffey Cycle Route / Options Assessment Summary

Furthermore, this document outlines that the Liffey Cycle Route is feasible and describes the Recommended Option arising from the assessment process.

The full Options Assessment Report is available on the National Transport Authority’s website at www.nationaltransport.ie

1.3 Previous Study

The previous study for the Liffey Cycle Route proposed a number of options which were presented to the public for consideration.

These options included the following:

Ĥ Two-way cycle tracks, both along and parallel to the River Liffey Corridor;

Ĥ One-way cycle tracks, North and South of the River Liffey; and

Ellis Quay - Proposed:

Ĥ Various combinations of the above.

Ĥ Key pinch-points, for example, at Ellis Quay and Ushers Island;

Ĥ Bus capacity and operational requirements, particularly at the busy central section of the

Each proposed option faced specific alignment

and environmental concerns, including:

4

SECTION 1

Photomontage of proposed scheme along Ellis Quay showing the Liffey Cycle route between the footpath and bus lane at a different level to both, a new boardwalk outside the quay wall, continuous bus and traffic lanes along the Quays, and enhanced footpaths.

Liffey Cycle Route / Options Assessment Summary

corridor along Bachelors Walk and Aston Quay; and

Ĥ Heritage constraints, including the need to respect and avoid historic bridge parapets.

On foot of the concerns raised through the consultation process, no recommended option emerged from the earlier study.

1.4 Project Objectives:

The need for the Liffey Cycle Route was identified in the Greater Dublin Area [GDA] Cycle Network Plan (2013), and is incorporated into the approved National Transport Authority Greater Dublin Area Transport Strategy (2016

– 2035) and the Dublin City development Plan 2016 - 2022.

The objectives of the Liffey Cycle Route are:

the north-south direction. The study area is wide enough to allow for the consideration of on-line

options and parallel routes.

To be consistent with the National Cycle Policy Framework and the NTA’s Transport Strategy for the Greater Dublin Area (2016 – 2035); and

To comply with the NTA’s National Cycle Manual and the principles within.

Ĥ

Ĥ

Ĥ To provide a safe, continuous cycle facility in both directions through the study area;

1.5 Study Area:

The study area includes the area along the Liffey Corridor between the Phoenix Park and the

Tom Clarke East Link Bridge in the east-west direction and between North Kings Street /

Parnell St and Dame Street / Thomas Street in

5

SECTION 1

Photo of a smiling cyclist

Liffey Cycle Route / Options Assessment Summary

Cabra

Phibsborough

Fairview

Clon

East Wall

Stoneybatter

Smithfield

Docklands

Townsend St

Ringsend

Irishtown

Portobello

Sandymount

Dolphin’s Barn

Ballsbridge

Ranelagh

Rathmines

Harold’s Cross

mnagh

Figure 1.1 Study Area

6

SECTION 1

Shelbourne Rd

Tritonville Rd

A map showing the study area from Islandbridge to the west and the Docklands to the east, from Stoneybatter/Phibsborough/East Wall to the north and Rialto/St. Stephen's Green/Ringsend to the south.

Liffey Cycle Route / Options Assessment Summary

02 DESIGN PRINCIPLES AND ASSUMPTIONS

7

SECTION 2

Stock image of cyclist travailing along a busy urban street

Liffey Cycle Route / Options Assessment Summary

  - National Cycle Manual

The design of the Recommended Option is compliant with the principles set out in the National Cycle Manual.

  - Current Traffic System:

The Liffey Quays form an integral part of the city’s traffic network, serving the core city centre with various bridges connecting the north and south city. It is a primary access and servicing corridor for businesses in Dublin City Centre. Traffic movement along this corridor is generally tidal in nature, that is, the predominant movement is towards the city centre in the morning and away from the city centre in the evening.

In addition, the North and South Quays provide an essential public transport function, catering for numerous bus routes into and out of the city centre. At present, Bachelors Walk accommodates in excess of 370 buses (c.30,

000 passengers) in the AM peak (7am to 10am).

favour of pedestrian, cycle, bus, and public realm improvements in compliance with current transport planning policy.

2.3 Environmental Issues:

The River Liffey Corridor has a number of significant landmarks, mature trees and environmental designations along its length. These include various historic buildings, bridges and street furniture that will necessitate further assessment during the development of this Recommended Option. The Recommended Option for the proposed Liffey Cycle Route seeks, insofar as possible, to enhance the

urban environment to the benefit of

the city and its occupants.

Cycling numbers now exceed 1,000 users during the AM peak (even in the absence of formal cycling facilities), and now constitute a greater percentage

of road users than car drivers.

The Recommended Option seeks to improve the public realm and pedestrian environment along the route, where practicable. New boardwalks will be provided to enhance the space available to pedestrians at various locations along the Recommended Option.

The Recommended Option for the Liffey Cycle Route recognises the need to maintain private car access to the core city centre, but existing provision for car parking will be reassigned in

8

SECTION 2

Liffey Cycle Route / Options Assessment Summary

Ormond Quay Lower -Proposed

9

SECTION 2

Photomontage of the proposed scheme along Ormond Quay Lower with the proposed Liffey Cycle route along the quay wall, showing the cyclist's interaction with the junction to Swifts Row/Jervis Street, a new boardwalk outside the quay wall, continuous bus and traffic lanes, and enhanced footpaths.

2

1. Liffey Cycle Route – Options Assessment Report

Liffey Cycle Route / Options Assessment Summary

03 OPTIONS CONSIDERED

10

SECTION 3

Stock Image of traffic signals

Liffey Cycle Route / Options Assessment Summary

3.1 Cycling Options:

The Liffey Cycle Route

– Options Assessment Study considered the feasibility of all possible options through the study area. This included on-line options along both River Quays and parallel routes to the North and South. It also includes options

that on first inspection may not readily lend themselves to the receiving environment but might form a viable solution if combined with others.

This included options that involve sharing with the Luas, options for the diversion of some modes from the river corridor, and options

for changes to the existing traffic circulation arrangement along the River Liffey. The Options Assessment Report describes all feasible

options in detail.

Inns Quay / Four Courts - Proposed

11

SECTION 3

Photomontage of the proposed scheme along Inns Quay outside the Four Courts, showing the interaction of the Liffey Cycle route with bus stops. A separate 3m wide platform will be provided for all bus stops, with cyclists cycling behind the platform and uncontrolled crossing points for pedestrians to cross the cycle track to the bus stop platform. Continuous bus and traffic lanes along Inns Quay and enhanced footpaths.

Liffey Cycle Route / Options Assessment Summary

04

METHODOLOGY AND APPRAISAL

12

SECTION 4

Stock image of cycle facilities, looking

at the pavement and road markings

mithfield

neybatter P

e  c

a   l

l    l

a

St	tSt  mi

Lw   Talbo       A

r

Sherif St Upr

bra	R

Rd

P

d	h

rcularR          R	b	B            Ec

i

s

NorthCi	at	o	er              cle

b

h	r	k	s

d	o	e

Blackh	o	u	le

w	g	y

orse	n	h	R

Av	R	R	d

e	d	d

t   S

Pr	y

Liffey Cycle Route / Options Assessment Summary

Section 1:

Phoenix Park to Father Mathews Bridge.

Section 2:

Father Mathews Bridge to O’Connell Bridge.

Section 3:

O’Connell Bridge to Matt Talbot Memorial Bridge.

Section 4:

Dublin’s Docklands.

  - Sections

The Liffey Cycle Route encompasses an area from the Phoenix Park in

the West to the Tom Clarke East Link Bridge in the East, a distance of almost 5km and travels through one of the most congested and densely populated areas of the city. The character and function of the river corridor varies along its length,

from the peri-urban western section, through the historic and core city centre, to the recent city expansion through the docklands. The route has therefore been considered in four sections.

  - Appraisal:

Each section was appraised in line with the requirements of the Common Appraisal Framework (CAF). This standardised process is used in transport assessment projects in Ireland, which considers relevant factors under five categories, which are:

Economy; Safety; Environment;

Accessibility and Social Inclusion; and

Integration.

Ĥ

Ĥ

Ĥ

Ĥ

Ĥ

Ĥ

Ĥ

Ĥ

The application of the above process identified

the Recommended Option.

Ĥ

East Wall

SECTION 3

SECTION 4

Sto

SECTION 1

SECTION 2

S

Docklands

Townsend St

Ringsend

Irishtown

Portobello

13

SECTION 4

rne Rd

Tritonville Rd

A map of the Liffey Cycle Route showing the four sections.

Liffey Cycle Route / Options Assessment Summary

05

RECOMMENDED OPTION

14

SECTION 5

Stock Image of 2 cyclists traveling side by side from the rear.

Liffey Cycle Route / Options Assessment Summary

The Recommended Option for the Liffey Cycle Route from the Phoenix Park to the Tom Clarke East Link:

In the western section of the route, over a length of 2.0km the cycle tracks will be located on the buildings side of the traffic lanes.

In the busiest central section over a length of 1.2km, the cycle tracks will be located on the river side of the traffic lanes so

as to avoid very busy

Ĥ

Ĥ

Aston Quay - Proposed:

15

SECTION 5

Photomontage of the proposed scheme along Aston Quay showing the Liffey Cycle route following the quay wall, a new boardwalk outside the quay wall, continuous bus and traffic lanes along the Quays, and enhanced footpaths. The existing trees along the quay wall will be kept.

Liffey Cycle Route / Options Assessment Summary

Section 1 – Phoenix Park to Father Mathews Bridge (Church / Bridge St):

The Recommended Option for Section 1 includes a with-flow segregated cycle track on the buildings side on both the North and South sides of the Liffey. This option includes connections to the Phoenix Park along Parkgate St, accommodating the current heavy demand towards the western suburbs. General traffic and buses will remain on the Quays for this entire section with pedestrians being accommodated on new boardwalks where necessary.

Section 3 – O’Connell St to Matt Talbot Memorial Bridge:

The Recommended Option for Section 3 continues river side as far as the Matt Talbot Memorial Bridge.

bus stops on the buildings side of the road.

The final eastern section over a distance of 1.8km through the Docklands will consist of two- way cycle tracks alongside the river on both sides.

Ĥ

Section 4 – Dublin Docklands:

The Recommended Option for the Docklands section is for a two-way cycle facility along the Campshires on each side of

the river.

Section 2 – Father Mathews Bridge to O’Connell Street:

The scheme will provide full segregation of cyclists from other traffic along its length.

This city centre section accommodates a large numbers of pedestrians, cyclists,

buses and other vehicles and thus requires the cyclist facilities to be on the river side to avoid interactions at the busy bus stops

along Bachelors Walk and Aston Quay.

16

SECTION 5

Liffey Cycle Route / Options Assessment Summary

17

SECTION 5

Stock photo of view over the River Liffey to the

Conference Centre and Samuel Beckett Bridge

2

1. Liffey Cycle Route – Options Assessment Report

Liffey Cycle Route / Options Assessment Summary

06 NEXT STAGE

18

SECTION 6

Stock image of Dublinbikes on the North Campshires.

Liffey Cycle Route / Options Assessment Summary

  - Public Engagement

It is proposed to undertake a

non-statutory public consultation for the project to seek the Public’s views relating to this proposal. These views will be considered in any proposals that are advanced to a formal statutory approval process.

  - Cost

As the scheme cost estimate exceeds the

€20m Public Spending Code (Department

of Public Expenditure and Reform) threshold, a detailed business case is required and will be prepared for the scheme at a later stage.

Bachelor’s Walk - Proposed:

19

SECTION 6

Photomontage of the proposed scheme along Bachelors' Walk, showing the Liffey Cycle route following along the quay wall, the existing boardwalk outside the quay wall, bus lanes and a traffic lane, and enhanced footpaths.

2

1. Liffey Cycle Route – Options Assessment Report

Liffey Cycle Route / Options Assessment Summary

07 RECOMMENDED OPTION – SCHEME DRAWINGS

20

SECTION 7

Stock Ariel Image of Dublin City Centre looking towards Heuston Station

la	oa

ck	th

ho	Rd

rs    e

Av

e

Navan

Rd	R

at oa

Rd	th

in	R

ph

Ne

Liffey Cycle Route

Liffey Cycle Route / Options Assessment Summary

7.1 Index Map

Drumcondra

Marino

Cabra

Phibsborough

Fairview

East Wall

Stoneybatter

Smithfield

Docklands

Townsend St

Ringsend

Irishtown

Legend:

Liffey Cycle Route Sections 1, 2 & 3 Proposal

Refer to BusConnects Proposals (www.busconnects.iPeo)rtobello

Dolphin’s Barn

Sandymount

Ballsbridge

21

SECTION 7

Shelbourne Rd

Tritonville Rd

The Map Index showing drawing numbers and locations along the proposed Liffey Cycle Route

Liffey Cycle Route / Options Assessment Summary

Map 1: Scheme Drawings

© Ordnance Survey Ireland Government of Ireland. All rights reserved. Licence Number EN 0082119 National Transport Authority.

Mh

H

H

Mh

H

Mh

Mh

Mh

M0NTPELIER COURT

Mh

BS

13.4

Mhs

Mh

H

Mhs

Mhs

Mhs

15.5

Mhs

H

H

Mhs

P

12.4

H

H

BUS

Stop 1474

*

Parkgate St

Hotel

Mh

Mh

LINK TO CHESTERFIELD AVENUE

& PHEONIX PARK CYCLE FACILITIES

H

BM 11.39

Mh

REARRANGED

VEHICULAR Tk

P

ACCESS

Mhs

H

9.8

Mh

10.9

Mhs

Mh

8.0

Mhs

PARKGATE STREET

CONYGHAM ROAD

H

Mhs

Mh

H

Mhs

LB

TKS

Mhs

ED Bdy

Mh

Mhs

7.0

Mh

Mh

Mh

5.1

H

Mhs

PARKGATE STREET

Mhs

8.7

H

Mhs

CHAPELIZOD

H

us Depot

CONYGHAM ROAD BUS DEPOT

Mh

7.9

Mhs

Mhs

BUS

Stop 7078

*

Parkgate St

Warehouse

LEGEND:

Bus Depot

CYCLE TRACK CYCLE LANE BUS LANE FOOTPATH

GENERAL TRAFFIC LUAS

EXISTING BOARDWALK NEW BOARDWALK EXISTING KERB LINE GRASS AREA / VERGE SHARED AREA

BUS STOP LOCATIONS

EXISTING BOUNDARY

PROPOSED NEW BOUNDARY (POSSIBLE LAND ACQUISITION)

NEW TREE (SUBJECT TO FURTHER INVESTIGATIONS)

EXISTING TREE

2.6m

1.75m

3m

3m

3.255m

2m

2.95m

Varies

Varies

2m

3m

3m

3m

3m

2m

3.318m

00

00

1234

1234

Destination	00

Destination	00

1234

1234

19 D ---

19 ---

19 D ---

19 D ---

RIVER LIFFEY

2.6m

EXISTING FOOTPATH

3.355m

GENERAL TRAFFIC LANE

3.15m

GENERAL TRAFFIC LANE

4.5m

GENERAL TRAFFIC LANE

2m

CYCLE LANE

2.95m

EXISTING FOOTPATH

Varies

4.115m

3.5m

2.9m

2.9m

3m

2.664m

EXISTING FOOTPATH

EXISTING FOOTPATH

GENERAL TRAFFIC LANE

GENERAL TRAFFIC LANE

GENERAL

GENERAL	GENERAL

TRAFFIC LANE TRAFFIC LANE TRAFFIC LANE

TREE TO BE REMOVED

EXISTING CROSS SECTION

EXISTING CROSS SECTION

P

BICYCLE PARKING

SECTION A-A CONYGHAM ROAD

SECTION B-B PARKGATE STREET

DUBLIN BIKES STATION

LÁNA BUS

EXCEPT BUSES

LÁNA BUS

SECTION 7

A

A

ROAD

CRIMINAL COURT OF JUSTICE

PHOENIX PARK

Proposed scheme between Conygham Road and Parkgate Street showing the Liffey Cycle route one-way on both sides of the road, the interaction with bus stops and enhanced footpaths.

Liffey Cycle Route / Options Assessment Summary

Map 2: Scheme Drawings

© Ordnance Survey Ireland Government of Ireland. All rights reserved.

Licence Number EN 0082119 National Transport Authority.

REARRANGED VEHICULAR ACCESS

P

THE CROPPY'S ACRE MEMORIAL PARK

BUS

Stop 1474

*

Parkgate St

P

P

BUS

BUS

BUS

Stop 7078

*

Parkgate St

BUS

RIVER LIFFEY

BUS PRIORITY SIGNAL

LOCAL LANE WIDENING FOR HGV TURNING MOVEMENTS

NEW SIGNALISED ENTRANCE TO ST. JAMES GATE BREWERY

P

RIGHT TURN STACKING LANE FOR CHESTERFIELD AVENUE BOUND CYCLISTS

LEGEND:

CYCLE TRACK

CYCLE LANE BUS LANE FOOTPATH

GENERAL TRAFFIC LUAS

EXISTING BOARDWALK NEW BOARDWALK EXISTING KERB LINE GRASS AREA / VERGE SHARED AREA

BUS STOP LOCATIONS

EXISTING BOUNDARY

PROPOSED NEW BOUNDARY (POSSIBLE LAND ACQUISITION)

NEW TREE (SUBJECT TO FURTHER INVESTIGATIONS)

EXISTING TREE

BUS

Stop  4319

*

Heuston Station

BUS

Stop  4320

*

Heuston Station

BUS

Stop  4425

*

Heuston Station

TREE TO BE REMOVED

P

BICYCLE PARKING

DUBLIN BIKES STATION

SECTION 7

EXCEPT BUSES

BUS

BUS

BUS

LÁNA BUS

FRANK SHERWIN BRIDGE

HEUSTON STATION

RIVER LIFFEY

Proposed scheme between Parkgate Street and Croppy's Acre Memorial Park showing the Liffey Cycle route one-way on on the north and south quays to follow the flow of traffic, cyclist connectivity between all roads south of the Frank Sherwin Bridge, improved bus facilities and enhanced footpaths.

Proposed scheme between Croppy's Acre Memorial Park and Rory O'Moore Bridge showing the Liffey Cycle route one-way on on the north and south quays to follow the flow of traffic, interaction with bus stops, improved bus facilities and enhanced footpaths.

Liffey Cycle Route / Options Assessment Summary

Map 3: Scheme Drawings

Mhs

© Ordnance Survey Ireland Government of Ireland. All rights reserved.

Platform

CLiivciel nDceefNenumceber EN 0082119 National Transport Authority.

BM 4.44

4.1

Mhs

H

Mh

Depot

Mh

Mhs

MhH

H

Mhs

Mh

FS

H

vil Defence

Mhs

Depot

M

H

The Croppy's Acre Memorial Park

33-56

THE CROPPY'S ACRE MEMORIAL PARK

H

BUS

Stop   NEW

*

Croppy's Acre

EXISTING BUS STOP RELOCATED

M7h0sm WEST TO CROPPY'S ACRE

Mh

P

Monument

BUS

Stop

Mhs

2.4

H

Mhs

WOLFE TONE QUAY

Mhs

H

3.2

Mhs

Mhs

Mhs

Mh

Mh BS

Mhs

4.7

Rory O'M

ED Bdy RIVER LIFFEY

BriPdge

BUS

BUS

VICTORIA QUAY

BUS

BUS PRIORITY SIGNAL

Mhs

2.6

BS

Mhs

Mh

Mhs

Mhs

2.4

2.4

Mhs

Mh

2.5

3.7

H

Mh

Mhs

Mhs

H

Mh

H

Mhs

H

Mhs

LOCAL LANE WIDENING FOR HGV TURNING MOVEMENTS

P

BUS

Stop 1447

ALL EXISTING BUS STOPS ON VICTORIA QUAY TO BE COMBINED AT GUINNESS STORE BUS STOP

St James's Gate Brewery

ST. JAMES'S GATE BREWERY

BUS

Stop 4407

*

Guinness Store

NEW SIGNALISED ENTRANCE TO ST. JAMES GATE BREWERY

WATLING STREET TO BECOME TWO-WAY FOR GENERAL TRAFFIC

Area under construction

H

LEGEND:

Viking Harbo

H

CYCLE TRACK CYCLE LANE BUS LANE FOOTPATH

GENERAL TRAFFIC LUAS

EXISTING BOARDWALK NEW BOARDWALK EXISTING KERB LINE GRASS AREA / VERGE SHARED AREA

BUS STOP LOCATIONS

EXISTING BOUNDARY

PROPOSED NEW BOUNDARY (POSSIBLE LAND ACQUISITION)

NEW TREE (SUBJECT TO FURTHER INVESTIGATIONS)

EXISTING TREE

PROPOSED CROSS SECTION

PROPOSED CROSS SECTION

1.8m

2m

2.5m

3m

3m

3m

1.9m

4.15m

2m

3m

3m

3m

1.9m

Mhs

00

00

Mh

1234

1234

Tks

Ramp

19 D ---

19 D ---

2.19m

2.75m

3.1m

3m

3m

1.91m EXISTING

1.88m

EXISTING FOOTPATH

3.65m

3m

3m

3.5m

2.17m

EXISTING FOOTPATH

1.1m

EXISTING CYCLE BUS LANE FOOTPATH LANE

GENERAL TRAFFIC LANE

GENERAL TRAFFIC LANE

GENERAL

BUS LANE

GENERAL TRAFFIC LANE

GENERAL TRAFFIC LANE

GENERAL TRAFFIC LANE

The Maltings (Apts)

Tk

P

TRAFFIC LANE FOOTPATH

TREE TO BE REMOVED

SECTION C-C VICTORIA QUAY

SECTION D-D WOLFE TONE QUAY

BICYCLE PARKING

DUBLIN BIKES STATION

2-20

EXCEPT BUSES

BUS

SECTION 7

EXCEPT BUSES

ISLANDISLAND

Liffey Cycle Route / Options Assessment Summary

Map 4: Scheme Drawings

© Ordnance Survey Ireland Government of Ireland. All rights reserved. Licence Number EN 0082119 National Transport Authority.

91-92

79

80-81

EXISTING BUS STOP RELOCATED 70m WEST TO CROPPY'S ACRE

BUS

BUS

Stop NEW

*

Croppy's Acre

Stop

EXISTING STREET BOLLARDS TO BE REMOVED ON ELLIS QUAY TO ACCOMMODATE NEW CYCLE FACILITIES

9-7

3

NO LEFT TURN

ELLIS

QUAY

HWM

P

RIVER LIFFEY

ED Bdy

RIVERSIDE-ROADSIDE FOOTPATH CLOSED

NEW BOARDWALK TO ACCOMMODATE PEDESTRIANS

USHER'S

9

WATLING STREET TO BECOME TWO-WAY FOR GENERAL TRAFFIC

BUS

Stop  NEW

*

Bridgefoot St.

BUS

Stop 4407

*

Guinness Store

3t

3t

LEGEND:

CYCLE TRACK CYCLE LANE BUS LANE FOOTPATH

GENERAL TRAFFIC LUAS

EXISTING BOARDWALK NEW BOARDWALK EXISTING KERB LINE GRASS AREA / VERGE SHARED AREA

BUS STOP LOCATIONS

EXISTING BOUNDARY

PROPOSED NEW BOUNDARY (POSSIBLE LAND ACQUISITION)

NEW TREE (SUBJECT TO FURTHER INVESTIGATIONS)

EXISTING TREE

PROPOSED CROSS SECTION

0.8m

PROPOSED CROSS SECTION

3.3m

PROPOSED CROSS SECTION

0.2m

3m

4m

3m

2.8m

3m

2m

2.8m

4m

1.5m

3m

1.5m

CYCLE TRACK

1.6m FOOTPATH

1.8m

2.8m

2.8m

1.2m

EXISTING QUAY WALL

EXISTING QUAY WALL

Destination	00

Destination	00

1234

1234

19 D ---

19 D ---

1.8m

EXISTING FOOTPATH

4.21m

2.59m

1.8m

3m

2.9m

3.75m

2.945m

EXISTING BOLLARD LINE

2.2m

EXISTING FOOTPATH

TRAFFIC LANE	EXISTING

EXISTING       GENERAL	GENERAL	EXISTING FOOTPATH  TRAFFIC LANE    TRAFFIC LANE	BUS LANE

EXISTING CROSS SECTION

EXISTING FOOTPATH

(ONE-WAY)

FOOTPATH

1.5m	3.55m

EXISTING GENERAL FOOTPATH TRAFFIC LANE

3.55m BUS LANE

EXISTING CROSS SECTION

SECTION F-F USHER ISLAND

TREE TO BE REMOVED

SECTION E-E WATLING STREET

EXISTING CROSS SECTION

P

BICYCLE PARKING

SECTION G-G ELLIS QUAY

DUBLIN BIKES STATION

72

8788 89

82-84

HWM

SECTION 7

LÁNA BUS

BUS

LÁNA BUS

BUILDING FACADE

BUILDING FACADE

JOHN

LÁNA BUS

BUILDING FACADE

0.125m

0.05m

0.05m

BUILDING FACADE

Proposed scheme between Rory O'Moore Bridge and Mellowe's Bridge showing the Liffey Cycle route one-way on on the north and south quays to follow the flow of traffic, interaction with bus stops, improved bus facilities, new boardwalks on Ellis Quay and Usher Island, and enhanced footpaths.

Liffey Cycle Route / Options Assessment Summary

Map 5: Scheme Drawings

Proposed scheme between Mellowe's Bridge and Father Matthew Bridge showing the Liffey Cycle route one-way on on the north and south quays to follow the flow of traffic, interaction with bus stops, improved bus facilities, and enhanced footpaths.

© Ordnance Survey Ireland Government of Ireland. All rights reserved. Licence Number EN 0082119 National Transport Authority.

LANE

EXISTING BUS STOP RELOCATED 120m EAST TO INNS QUAY

NO LEFT TURN

BUS

P

P

Stop

BUS

Stop 7453

*

Ocean House

EXISTING STREET BOLLARDS TO BE REMOVED ON ELLIS QUAY TO ACCOMMODATE NEW CYCLE FACILITIES

ARRAN QUAY

ARRAN QUAY

RIVERSIDE-ROADSIDE FOOTPATH CLOSED

NEW BOARDWALK TO ACCOMMODATE PEDESTRIANS

RIVER LIFFEY

USHERS QUAY

USHERS QUAY

P

BUS

Stop 1445

*

Usher St

BUS

Stop   NEW

*

Bridgefoot St.

1

LEGEND:

1-12

CYCLE TRACK CYCLE LANE BUS LANE

FOOTPATH GENERAL TRAFFIC LUAS

EXISTING BOARDWALK NEW BOARDWALK EXISTING KERB LINE GRASS AREA / VERGE SHARED AREA

BUS STOP LOCATIONS

EXISTING BOUNDARY

PROPOSED NEW BOUNDARY (POSSIBLE LAND ACQUISITION)

NEW TREE (SUBJECT TO FURTHER INVESTIGATIONS)

EXISTING TREE

2.5m

3m

3m

2m

5.205m

1.165m

0.3m

3m

3m	2.001m

2.185m

1.5m 0.6m

1.835m

3m

3m

EXISTING QUAY WALL

EXISTING QUAY WALL

Destination	00

Destination	00

1234

1234

NEW SIGNALISED JUNCTION

19 D ---

19 D ---

3m

3m

4.1m EXISTING BUS LANE

3.8m EXISTING FOOTPATH

1.625m EXISTING

3.335m

3.485m

1.79m

1.8m

1.165m EXISTING FOOTPATH

GENERAL

GENERAL

GENERAL

GENERAL EXISTING TRAFFIC LANE FOOTPATH

EXISTING TRAFFIC LANE TRAFFIC LANE

FOOTPATH TRAFFIC LANE

FOOTPATH

TRAFFIC LANE

BUS LANE

FOOTPATH

TREE TO BE REMOVED

EXISTING CROSS SECTION

EXISTING CROSS SECTION

SECTION J-J USHER QUAY

EXISTING CROSS SECTION

SECTION K-K ARRAN QUAY

P

BICYCLE PARKING

SECTION H-H MELLOWES BRIDGE

DUBLIN BIKES STATION

STOP

Local Governmant House 39

79

Bridgefoot Peak

Emmet Place

Mellows Quay

USHER STREET

BUILDING FACADE

LÁNA BUS

LÁNA BUS

BUS

BUS

LÁNA BUS

LÁNA BUS

SECTION 7

J

J

ARRAN STREET

LÁNA BUS

BUS

STURSHEER ESTRTEET

LÁNA BUS

85-390S

BUS

K

K

LÁNA BUS

LÁNA BUS

LINCOLNLLINANCEOLN

LÁNA BUS

LÁNA BUS

SAINT AUGUSTINE STREET

CHURCH STREET

3.415m GENERAL

4.645m EXISTING

2.185m EXISTING

PETROL STN.

USHERS QUAY

Liffey Cycle Route / Options Assessment Summary

Map 6: Scheme Drawings

© Ordnance Survey Ireland Government of Ireland. All rights reserved. Licence Number EN 0082119 National Transport Authority.

H

Mh

Arran Quay Apts

Security Hut (postal address)

H

H

Mhs

Mh

Mhs H

Mh

Mh

Mh

Mh

EXISTING BUS STOP RELOCATED 120m EAST TO INNS QUAY

Mhs

H

Mhs

H

Mhs

BUS

B

Stop

P

H

FOUR COURTS

Mh

Mhs

Mh

Mh

Mh

Mh

The Ormond Buil

(31-36)

P

Mh

Mh

BUS

Mh

Mh

Mhs

Stop NEW

*

Inns Quay

H

H

BUS

Stop  1478

*

Inns Quay

Mh

P

Mh

Mhs

Mh

Mhs

Mh

P

Mhs

Mh

H

Mhs

Mh

Mh

Mh

Mh

Mh

Mh

H

Mhs

Mh

Mhs

Father Mathew Bridge

Mh

Mhs

Mh

Mhs

Mh

Mhs

Mh

Mh

O'Donovan Rossa Bridge

Mh

CYCLE FACILITIES TRANSITION FROM BUILDING SIDE TO RIVER SIDE (CONTROLLED CROSSING)

H

RIVER LIFFEY

BS

BM 4.69 BS

H

Mhs

BM 3.51

PO

Mh

CYCLE FACILITIES TRANSITION FROM RIVER SIDE TO BUILDING SIDE (CONTROLLED CROSSING)

Mh

Mhs

Mhs

Mh

Mh

Mhs

Mhs

Mh Mh

Mh

Bridge Gate (Site of)

Mh

Mh

H

Mh

Mh

Mh

Mh

Mhs

Mhs

Mh

H

Mhs

Merchant's

Mh

Mhs

LEFHToTUuRsNeMOVEMENT RMETeArIcNEhDant's

Hall

Mh

H

P

H

Mh

Mh

Mh

Mhs

H

H

Mh

Mh

BUS

Stop 1444

*

Merchants Quay

P

Mhs

Hall

DUBLIN CITY COUNCIL OFFICES

LEGEND:

Marshalsea

CYCLE TRACK

CYCLE LANE

BUS LANE FOOTPATH GEMNEhRAL TRAFFIC LUAS

EXISTING BOARDWALK NEW BOARDWALK EXISTING KERB LINE GRASS AREA / VERGE SHARED AREA

BUS STOP LOCATIONS

y

k

3m

3m

3m

3m

2.5m

1.75m

4.4m

Mh

H

Mhs

EXISTING QUAY WALL

Destination	00

Destination	00

1234

1234

St Au

Apts

EXISTING BOUNDARY

PMROhPsOSED NEW BOUNDARY (POSSIBLE LAND ACQUISITION)

19 D ---

19 D ---

H

H

NEW TREE (SUHBJECT TO FURTHER INVESTIGATIONS)

EXISTING TREE

TREE TMO BhE REMOVED BICYCLE PARKING

1.8m

2.155m

3m

3m

3m

1.2m CYCLE

6.495m

Mh

EXISTING   PARKING         GENERAL	GENERAL

GENERAL

EXISTING FOOTPATH

H

TRAFFIC LANE LANE

FOOTPATH

TRAFFIC LANE TRAFFIC LANE

Mh

H

EXISTING CROSS SECTION

SECTION L-L INNS QUAY

P

Mhs

H

H

Mhs

Mh

Mhs

Mh

DOULBIVLEINRBBIKOENSDSSTTARTEIOENT

Mhs

SECTION 7

LÁNA BUS

BUILDING FACADE

Proposed scheme between Father Matthew Bridge and O'Donovan Rossa Bridge showing the Liffey Cycle route one-way on on the north and south quays to follow the flow of traffic, interaction with bus stops, improved bus facilities, and enhanced footpaths.

Proposed scheme between O'Donovan Rossa Bridge and Grattan Bridge showing the transition of the Liffey Cycle route from following the footpaths on the building side of the Quays to following inside the quay walls, a new boardwalk outside the quay wall on Essex Quay, improved bus facilities, existing tree retention and enhanced footpaths.

Liffey Cycle Route / Options Assessment Summary

Map 7: Scheme Drawings

© Ordnance Survey Ireland Government of Ireland. All rights reserved. Licence Number EN 0082119 National Transport Authority.

BUS

Stop 1479

*

Ormand Quay

P

168

BUS

Stop 1480

*

Ormond Quay (House)

P

BUS

Stop   NEW

*

Ormand Quay (Arran St)

P

25

1

5

UPPER

QUAY

CYCLE FACILITIES TRANSITION FROM BUILDING SIDE TO RIVER SIDE (CONTROLLED CROSSING)

RIVERSIDE-ROADSIDE FOOTPATH CLOSED

NEW BOARDWALK TO ACCOMMODATE PEDESTRIANS

RIVER LIFFEY

RIVER LIFFEY

E

CYCLE FACILITIES TRANSITION FROM RIVER SIDE TO BUILDING SIDE (CONTROLLED CROSSING)

RIVERSIDE-ROADSIDE FOOTPATH CLOSED

NEW BOARDWALK TO ACCOMMODATE PEDESTRIANS

RIVERSIDE-ROADSIDE FOOTPATH CLOSED

NEW BOARDWALK TO ACCOMMODATE PEDESTRIANS

DUBLIN CITY COUNCIL OFFICES

P

22

18

P

BUS

Stop 1443

*

Exchange Street

MAINTAIN EXISTING NO LEFT TURN

24

LEGEND:

CYCLE TRACK CYCLE LANE BUS LANE FOOTPATH

GENERAL TRAFFIC LUAS

EXISTING BOARDWALK NEW BOARDWALK EXISTING KERB LINE GRASS AREA / VERGE SHARED AREA

BUS STOP LOCATIONS EXISTING BOUNDARY

0.6m MIN

2.5m

2m

3m

3m

3m

5m

2m

2m

3m

3m

3m

5.348m

EXISTING QUAY WALL

Destination	00

Destination	00

EXISTING QUAY WALL

Destination	00

Destination	00

1234

1234

1234

1234

PROPOSED NEW BOUNDARY (POSSIBLE LAND ACQUISITION)

NEW TREE (SUBJECT TO FURTHER INVESTIGATIONS)

EXISTING TREE

19 D ---

19 D ---

19 D ---

19 D ---

1.5m

EXISTING FOOTPATH

4.48m

GENERAL TRAFFIC LANE

3.85m

GENERAL TRAFFIC LANE

4.65m BUS LANE

2.12m

EXISTING FOOTPATH

1.8m

2.2m

3m

3m

3m

2.3m

3.1m

EXISTING PARKING GENERAL

GENERAL TRAFFIC LANE

BUS LANE           PARKING          EXISTING

FOOTPATH

TREE TO BE REMOVED

FOOTPATH

TRAFFIC LANE

P

EXISTING CROSS SECTION

SECTION M-M WOOD QUAY

BICYCLE PARKING

EXISTING CROSS SECTION

SECTION N-N ORMOND QUAY (WEST)

DUBLIN BIKES STATION

9

62

36

4

SQUARE

7-8

3

1A

ORMOND QUAY

ORMOND QUAY

ED Bdy

20

ESSEX S

PARLIAMENT STREET

CAPEL STREET

Grattan

Bridge

LÁNA BUS

BUS

FISHAMBLE

BUS

ORMOND MKT

Ormond

House

BUS

ET

WESTCHARLES ST. WEST

PLACE

CHANCERY PLACE

WINETAVERN STREET ED Bdy

SECTION 7

BUS

BUS

N

N

BUS

LÁNA BUS

168

Liffey Cycle Route / Options Assessment Summary

Map 8: Scheme Drawings

© Ordnance Survey Ireland Government of Ireland. All rights reserved. Licence Number EN 0082119 National Transport Authority.

31

12

BUS

Stop 7622

*

Liffey St

P

EXISTING BUS STOP RELOCATED 200m EAST TO BACHELOR'S WALK

1

5

BUS

Stop

36-37

MINOR ALTERATIONS TO HA'PENNY BRIDGE WING WALLS

RIVERSIDE-ROADSIDE FOOTPATH CLOSED

NEW BOARDWALK TO ACCOMMODATE PEDESTRIANS

RIVER LIFFEY

ED Bdy

MINOR ALTERATIONS TO HA'PENNY BRIDGE WING WALLS

RIVERSIDE-ROADSIDE FOOTPATH CLOSED

NEW BOARDWALK TO ACCOMMODATE PEDESTRIANS

RIVERSIDE-ROADSIDE FOOTPATH CLOSED

NEW BOARDWALK TO ACCOMMODATE PEDESTRIANS

HWM

BUS

Stop 330

*

Crampton Quay

P

14-15

22

BUS

Stop

BUS

Stop 312

*

Wellington Quay

18

EXISTING BUS STOP RELOCATED ASTON QUAY & WELLINGTON QUAY

MAINTAIN EXISTING NO LEFT TURN

24

29

LEGEND:

ESSEX

CYCLE TRACK

CYCLE LANE BUS LANE FOOTPATH

GENERAL TRAFFIC

LUAS

0.6m

Varies

EXISTING BOARDWALK NEW BOARDWALK EXISTING KERB LINE GRASS AREA / VERGE SHARED AREA

BUS STOP LOCATIONS

EXISTING BOUNDARY

PROPOSED NEW BOUNDARY (POSSIBLE LAND ACQUISITION)

4m

2m

3m

3m

3m

3.915m

(600mm Min.)

4m

2m

3m

3m

3m

3.1m

EXISTING QUAY WALL

Destination	00

EXISTING QUAY WALL

Destination	00

Destination	00

1234

1234

1234

19 D ---

19 D ---

19 D ---

33N-3EW4 TREE (SUBJECT TO

FURTHER INVESTIGATIONS) EXISTING TREE

TR3E5E TO BE REMOVED

BICYCLE PARKING

1.7m	3m

EXISTING GENERAL FOOTPATH TRAFFIC LANE

3m

GENERAL TRAFFIC LANE

4.7m BUS LANE

3.3m

EXISTING FOOTPATH

4m

1.9m	3m

EXISTING GENERAL FOOTPATH TRAFFIC LANE

3m BUS LANE

3m

3.115m

1.5m

GENERAL CYCLE EXISTING

TRAFFIC LANE

LANE

FOOTPATH

EXISTING CROSS SECTION

P

EXISTING CROSS SECTION

SECTION O-O WELLINGTON QUAY

SECTION P-P ORMOND QUAY (EAST)

DUBLIN BIKES STATION

7-8

38

34

1A

28

QUAY

ORMOND QUAY

Liffey

20

21

SECTION 7

BUILDING FACADE

LIFFEY STREET

STRLOEWEERT

LOWER

34-35

SWIFT ROW

CRANE LANE

PARLIAMENT STREET

CAPEL STREET

PARLIAMENT

Grattan

Bridge

LÁNA BUS

LÁNA BUS

BUILDING FACADE

Proposed scheme between Grattan Bridge and the Ha'penny Bridge showing the Liffey Cycle route following inside the quay walls, new boardwalks on both quays outside the quay walls, improved bus facilities, some tree retention and some tree removal, and enhanced footpaths.

Mhs

Proposed scheme between the Ha'penny Bridge and O'Connell Bridge showing the Liffey Cycle route following inside the quay walls, new boardwalks on the south quays outside the quay walls, improved bus facilities, tree removal on the north quays and tree retention on the south quays, and enhanced footpaths.

Liffey Cycle Route / Options Assessment Summary

Map 9: Scheme Drawings

© Ordnance Survey Ireland Government of Ireland. All rights reserved.

Licence Number EN 0082119 National Transport Authority.

P

BUS

Stop 135351

*

Eden Quay

BUS

Stop 7622

*

Liffey St

BUS

Stop 315

*

Liffey St

BUS

Stop 316

*

Batchelors Walk

P

31

Boardwalk

EDEN QUAY

BUS

Stage

SIDE FOOTPATH CLOSED

Landing

BUS

MINOR ALTERATIONS TO HA'PENNY BRIDGE WING WALLS

RIVERSIDE-ROAD

BUS

NEW BOARDWALK TO ACCOMMODATE PEDESTRIANS

BUS PRIORITY SIGNAL

LOCAL CYCLE FACILITY WIDENING TO 3m FOR CROSSING CYCLISTS

RIVER LIFFEY

ED Bdy

ED Bdy

P

MINOR ALTERATIONS TO HA'PENNY BRIDGE WING WALLS

P

RIVERSIDE-ROADSIDE FOOTPATH CLOSED

NEW BOARDWALK TO ACCOMMODATE PEDESTRIANS

BUS

Stop

EXISTING BUS STOP RELOCATED TO D'OLIER STREET

ASTON QUAY

ASTON QUAY

CRAMPTON QUAY

BURGH QUAY

NO ENRTY

CRAMPTON QUAY

QUAY

1-3

BUS

Stop 330

*

Crampton Quay

BUS

Stop   NEW

*

*

BUS

Stop  328

*

Aston Place

BUS

Stop 7392

*

Aston Place

BUS

Stop  326

*

Aston Place

BUS

Stop 4720

*

Westmoreland Street

BUS

Stop 325

*

Westmoreland Street

BUS

Stop  329

*

Bedford Row

LEFT TURN MOVEMENT RETAINED

LEGEND:

CYCLE TRACK CYCLE LANE BUS LANE

FOOTPATH GENERAL TRAFFIC LUAS

EXISTING BOARDWALK NEW BOARDWALK EXISTING KERB LINE GRASS AREA / VERGE SHARED AREA

BUS STOP LOCATIONS

EXISTING BOUNDARY

PROPOSED NEW BOUNDARY (POSSIBLE LAND ACQUISITION)

NEW TREE (SUBJECT TO FURTHER INVESTIGATIONS)

EXISTING TREE

0.5m

4m

1.75m

3m

3m

3m

6.174m

Varies

(600mm Min.)

4m

2m

3m

3m

3m

4.5m

EXISTING QUAY WALL

Destination	00

Destination	00

EXISTING QUAY WALL

1234

1234

Destination

00

Destination

00

1234

1234

19 D ---

19 D ---

19 D ---

19 D ---

EXISTING

2.015m

3.15m

3.15m

4.65m

4.46m

3.95m

EXISTING FOOTPATH

3m

GENERAL TRAFFIC LANE

3m BUS LANE

3m BUS STOP

4.45m

EXISTING FOOTPATH

BOARDWALK	EXISTING GENERAL

FOOTPATH TRAFFIC LANE

GENERAL TRAFFIC LANE

BUS LANE

EXISTING FOOTPATH

EXISTING CROSS SECTION

TREE TO BE REMOVED

EXISTING CROSS SECTION

SECTION R-R BACHELOR'S WALK

P

SECTION Q-Q ASTON QUAY

BICYCLE PARKING

DUBLIN BIKES STATION

BACHELORS

( Apts )

WALK

BACHELORS

HWM

ORMOND QUAY

BACHELOR'S WALK

BACHELOR'S WALK

BACHELOR'S WALK

RIVER LIFFEY

BURGH QUAY

BUS

LÁNA BUS

BUS

BUS

BUS

O'CONNELL BRIDGE

LÁNA BUS

BUS

LÁNA

BUS

BUS

LÁNA BUS

BUS

BUS

BUS

BUS

BUS

LÁNA BUS

SECTION 7

LÁNA

BUS  BUS

Q

Q

BUS

BUS

BUS

BUS

LÁNA BUS BUS

BUS

BUS

BUILDING FACADE

EXCEPT BUSES

BUS

BUILDING FACADE

Proposed scheme between O'Connell Bridge and Butt Bridge showing the Liffey Cycle route following inside the quay walls, cyclist interaction with major junctions, extending the existing boardwalk on Eden Quay, improved bus facilities, some tree removal on the north quays and tree retention on the south quays, and enhanced footpaths.

Liffey Cycle Route / Options Assessment Summary

Map 10: Scheme Drawings

© Ordnance Survey Ireland Government of Ireland. All rights reserved. Licence Number EN 0082119 National Transport Authority.

HARBOUR COURT

OLD ABBEY STREET

P

BUS

Stop 135351

*

Eden Quay

LEFT TURN MOVEMENT RETAINED

BUS

Stop 7359 & NEW

*

Eden Quay

P

BUS

297

BUS

Stop  298

*

Liberty Hall

BUS

Stop  299

*

Liberty Hall

BUS

Stop  300

*

Liberty Hall

Stop

*

Marlborough St.

Astor Hall

NO  ENRTY

EDEN QUAY

11

P

32

23-24

EDEN QUAY

BoardWaElDkEN QUAY

P

EDEN QUAY

LOCAL CYCLE FACILITY WIDENING TO 3m FOR CROSSING CYCLISTS

EDEN QUAY

BUS

Stop 301

*

Marlborough Street

BUS

Stop 302

*

Marlborough Street

BUS

Stop 303

*

Marlborough Street

P

P

BURGH QUAY

BURGH QUAY

BURGH QUAY

P

BURGH QUAY

BUS

Stop  NEW

*

*

BUS

Stop  NEW

*

*

BUS

Stop 4494

*

Hawkins St.

BUS

Stop  339

*

Burgh Quay

LEFT TURN MOVEMENT RETAINED

BUS

Stop  326

*

Aston Place

BUS

Stop 4720

*

Westmoreland Street

BUS

Stop 325

*

Westmoreland Street

BUS

Stop  NEW

*

LEGEND:

CYCLE TRACK CYCLE LANE BUS LANE FOOTPATH

GENERAL TRAFFIC LUAS

EXISTING BOARDWALK NEW BOARDWALK EXISTING KERB LINE GRASS AREA / VERGE SHARED AREA

BUS STOP LOCATIONS

EXISTING BOUNDARY

PROPOSED NEW BOUNDARY (POSSIBLE LAND ACQUISITION)

NEW TREE (SUBJECT TO FURTHER INVESTIGATIONS)

EXISTING TREE

2.525m

2m

3m

3m

3m

7.04m

4m

3.45m

2m

3m

3m

3.3m

3m

3.275m

EXISTING QUAY WALL

EXISTING QUAY WALL

Destination	00

Destination	00

Destination	00

Destination	00

Destination	00

1234

1234

1234

1234

1234

19 D ---

19 D ---

19 D ---

19 D ---

19 D ---

EXISTING

3.24m

4m

3m

3.5m

2.5m TAXI RANK

3.275m

1.5m CYCLE LANE

2.226m

3.15m

3.15m

Varies

3.3m

1.5m CYCLE LANE

3m

2.915m

TREE TO BE REMOVED

BOARDWALK

EXISTING FOOTPATH

BUS LANE

GENERAL TRAFFIC LANE

BUS LANE

EXISTING FOOTPATH

GENERAL CHEVRONS BUS LANE TRAFFIC LANE

EXISTING CROSS SECTION

EXISTING GENERAL FOOTPATH TRAFFIC LANE

BUS STOP

EXISTING FOOTPATH

P

BICYCLE PARKING

EXISTING CROSS SECTION

SECTION S-S EDEN QUAY

SECTION T-T BURGH QUAY

DUBLIN BIKES STATION

34

ED Bdy

SECTION 7

NO ENRTY

LÁNA BUS

BUS

BUS

LÁNA BUS

LÁNA BUS

BUS

LÁNA BUS

LÁNA BUS

O'CONNELL BRIDGE

BUS

LÁNA BUS

S

S

LÁNA BUS

BUS

NO ENRTY

BUILDING FACADE

BUILDING FACADE

Proposed scheme between Butt Bridge and Talbot Memorial Bridge showing the Liffey Cycle route following inside the quay walls, cyclist interaction with major junctions, improved bus facilities, tree retention on both quays, and enhanced footpaths. East of Talbot Memorial Bridge along the Campshires, the Liffey Cycle Route will be a two-way facility on both quays as per Bus Connects proposals.

Liffey Cycle Route / Options Assessment Summary

Map 11: Scheme Drawings

© Ordnance Survey Ireland Government of Ireland. All rights reserved.

Licence Number EN 0082119 National Transport Authority.

V.H.I.

Custom House

Mh

House

Mhs

hurch

Statue

res)

Bank

H

Mhs

BUS

Stop   NEW

*

Custom House

BUS

Stop 407

*

Custom House

BUS

Stop   NEW

*

Custom House

BUS

Stop   NEW

*

Custom House

H

Mhs

LB

Mhs

LEFT TURN MOVEMENT RETAINED

Mhs

Liberty Hall

P

H

Mhs Mh

Mh

H

BUS

Stop  300

*

Liberty Hall

Mhs

Mh

BUS

Stop  299

*

Liberty Hall

H

BM

Mhs

Mh

4.67

Mhs

Mhs

CUSTOM

Mh

3.4

3.7

Mhs

Mh

Mhs

Mh

H

H

Mh

Mh

H

Mhs

Mh

Mhs

Mh

H

Mh

HWM

2.9

Mh

Mhs

s

Mhs

BM 4.00

P

Mh

BM 3.60

LÁNA BUS

DOCKLANDS

Talbot Memorial Bridge

BUSButt Bridge

Stop 303

*

Marlborough Street

BUS

Stop 302

*

Marlborough Street

Mh

Mh

P

P

HWM

GEORGE'S QUAY

BURGH QUAY

GBEMORG3E.9'S6QUAY

H

H

H

H

Mh

Mh

3.7

Mhs

Mh

Mh

Mh

Mh

H

3.2

3.2

Mhs

2.7

3.0

Mhs

Mh

Mh

Mhs

Mhs

Mh

H

H

Mh

Mhs

Mhs

STOP

Mhs

Mhs

BM 3.29

Mh

H

Mh

H

Mh

Mh

Mh H

BUS

Stop  339

*

Burgh Quay

2.8

BUS

Stop   NEW

*

TARA ST. STATION

BUS

Stop   NEW

*

George's Quay

BUS

Stop   NEW

*

George's Quay

BUS

BUS

Stop 135142

*

Georges Quay

BUS

Stop 1502

*

Tara Street Station

Mhs

Stop -

*

George's Quay

Apts

Mh

Mh

Platform

Mh

Exchange Court

LEGEND:

Mhs

Presbytery

CYCLE TRACK CYCLE LANE BUS LANE FOOTPATH

BM 3.56

Mh

Mh

GENERAL TRAFFIC H

LUAS

5m

2m

2.8m

3m

3m

3m

4.45m

H

7.1m

3m

1m

3m

3m

3m

7.815m

EXISTING BOARDWALK

7

8-9

NEW BOARDWALK

EXISTING KERB LINEMhs

h

Mhs

H Mh

MhGsRASS PAREOA /OVERLGEBEG SMThREET

SHARED AREA

BUS STOP LOCATIONS

EXISTING BOUNDARY

PROPOSED NEW BOUNDARY (POSSIBLE LAND ACQUISITION)

NEW TREE (SUBJECT TO FURTHER INVESTIGATIONS)

EXISTING TREE

H

Mhs

EXISTING QUAY WALL

Mhs

EXISTING QUAY WALL

Mhs

Mhs

00

Destination	00

Destination	00

1234

Destination

00

1234

1234

1234

Apollo House

Mh

19 ---

19 D ---

19 D ---

19 D ---

8.123m

4m

3.25m

3.3m

7.871m

7.541m

3m

3m

3.3m

1.5m CYCLE LANE

4.765m

House

Mh

1.5m CYCLE LANE

Car Park

H

Mh

TREE TO BE REMOVED

EXISTING FOOTPATH

CONTRAFLOW BUS LANE

GENERAL TRAFFIC LANE

BUS LANE

EXISTING FOOTPATH

EXISTING FOOTPATH

GENERAL TRAFFIC LANE

BUS LANE	BUS STOP

EXISTING FOOTPATH

P

BICYCLE PARKING

EXISTING CROSS SECTION

EXISTING CROSS SECTION

SECTION U-U GEORGE'S QUAY

SECTION V-V CUSTOM HOUSE QUAY

DUBLIN BIKES STATION

RIVER

LIFFEY

7

7A

LÁNA BUS

BUS

BUS

BUS

BUS

LUKE STREET

BUS

Ramp

LÁNA BUS

BERESFORD PLACE

TARA STREET

TARA ST

BUTT BRIDGE

BUS

CORN EXCHANGE PLACE

SECTION 7

BUS

BUS

V

V

U

BUS

U

BUS

BUILDING FACADE

BUILDING FACADE

IFSC

CUSTOM HOUSE

National Transport Authority Dún Scéine

Harcourt Lane Dublin 2

D02 WT20

www.nationaltransport.ie